| FORM 2 THE PATENTS ACT 1970 39 of 1970 & The Patent Rules 2003 COMPLETE SPECIFICATION (See sections 10 & rule 13) |  |  |
| --- | --- | --- |
| - TITLE OF THE INVENTION ## AN OVER-THE-FRAME BONDING TECHNIQUE FOR RUGGEDIZING COMMERCIAL OFF-THE-SHELF LIQUID CRYSTAL DISPLAY PANELS WITHOUT ANY INTERNAL DECONSTRUCTION AND A PROCESS OF MAKING THE SAME |  |  |
| - APPLICANT (S) |  |  |
| NAME | NATIONALITY | ADDRESS |
| Samtel Avionics Ltd | Indian | 2nd Floor, Plot No. 288-A, Udyog Vihar Phase-IV, Gurugram - 122015, Haryana, India. |
| - PREAMBLE TO THE DESCRIPTION |  |  |
| COMPLETE SPECIFICATION The following specification particularly describes the invention and the manner in which it is to be performed. |  |  |

ABSTRACT

A process for making ruggedization for commercial off-the-shelf (COTS) liquid crystal display (LCD) modules using a non-intrusive ruggedization method. This process discloses an over-the-frame bonding technique in which the original LCD module, including the LCD cell, electronics, optical stack, mechanical frame, and factory-integrated backlight, is retained intact without dismantling or reconfiguration. An optical clear adhesive with appropriate formulation is applied over and along the existing LCD frame to form a bonding interface for integrating additional protective and functional layers, such as front cover glass, touch panels, optical filters, transparent heaters, and environmental sealing elements. The adhesive is cured to provide mechanical reinforcement, environmental sealing and improved resistance to shock and vibration. The technique eliminates the need for under-the-frame bonding, internal edge bonding, or full-face bonding, thereby reducing manufacturing time, cost and risk while preserving factory calibration and alignment of COTS LCD module. The invention is particularly suited for rapid ruggedization of LCDs used in defense, industrial, transportation, and outdoor display applications.

Field of the Invention:

The present invention relates to ruggedization of Liquid Crystal Displays (LCDs) and, more particularly, to an over-the-frame bonding technique that enhances the mechanical, environmental, and functional performance of commercial off-the-shelf (COTS) LCD panels without altering or dismantling their original internal construction, including the LCD cell, electronics, optics, and backlight.

Background of the Invention:

Conventional ruggedization of LCDs often involves under-the-frame bonding techniques, wherein the original LCD module is dismantled. In such approaches, the LCD glass and LCD cell are retrieved, while the original electronic, mechanical, optical, and backlight components are discarded. A new ruggedized stack is then built using techniques such as edge bonding or full-face bonding, followed by integration of custom backlights, filters, and mechanical structures.

While effective, such methods are time-consuming, costly, and complex, requiring extensive rework and redesign of the LCD assembly.

There exists a requirement for a faster, less intrusive, and cost-effective ruggedization method that:

- Preserves the original LCD module

- Retains the factory-integrated backlight and electronics

- Avoids dismantling of the LCD internal stack

- Enables addition of protective and functional layers

Summary of the Invention:

The present invention discloses an over-the-frame bonding technique, wherein the original COTS LCD module, including its LCD cell, electronics, optical stack, and backlight, is retained intact.

An adhesive is dispensed along and over the original LCD frame of the COTS module, creating a bonding interface on which additional ruggedization elements are integrated. These elements may include:

- Front protective glass

- Touchscreen overlay

- Optical filters

- Transparent heaters

- Environmental sealing layers

The technique enables ruggedization without removal of the original backlight or internal components, thereby significantly reducing processing time, cost, and risk.

Detailed Description of the Invention:

In one embodiment, the method comprises the following steps:

- Selection of a COTS LCD Module:

The original LCD module, as supplied by the manufacturer, including the LCD glass, driver electronics, mechanical frame, optical layers, and backlight, is used without disassembly.

- Adhesive Dispensing Over the Original Frame:

A specially formulated adhesive is dispensed over and along the perimeter frame of the original LCD construction, without penetrating beneath the frame or disturbing the internal layers.

- Integration of Additional Functional Layers:

One or more functional or protective layers are bonded over the adhesive layer, including but not limited to:

  - Front cover glass

  - Touch-sensitive panel

  - EMI/IR/UV or neutral density filters

  - Transparent heater elements

- Curing and Sealing:

The adhesive is cured to form a mechanically robust and environmentally sealed assembly, resulting in a ruggedized LCD module.

The bonding interface distributes mechanical stress, enhances environmental resistance, and provides mechanical damping during shock and vibration events.

Key Differentiation from Under-the-Frame Bonding

Unlike under-the-frame bonding techniques, the present invention:

- Does not require removal of the LCD glass or cell

- Does not discard original electronics, optics, or backlight

- Does not involve full-face or internal edge bonding

- Preserves original factory calibration and alignment

Advantages of the Invention:

- Significantly reduced ruggedization time

- Lower manufacturing cost

- Minimal risk of LCD damage

- Retention of original backlight and electronics

- Suitable for rapid ruggedization of COTS LCDs

- Ideal for low-volume and quick-turn defence and industrial programs

Industrial Applicability:

The invention is applicable to rugged display systems used in defence vehicles, avionics, naval platforms, railway displays, industrial HMIs, and outdoor electronic equipment, where rapid deployment and reliability are critical.