# Invention Disclosure Write-up

## Title of the Invention

An Array-Based Backlight Scheme with Alternate Switching of Day and NVG LEDs for Direct and Edge Lighting Applications

## Field of the Invention

The present invention relates to backlight architectures for display systems, particularly for dual-mode day and Night Vision Goggle (NVG) compatible LCD displays. More specifically, the invention relates to a single printed circuit board (PCB)-based LED array supporting alternate switching of day and NVG LEDs, applicable to both direct lighting and edge lighting schemes, while maintaining uniform luminance and common mechanical real estate.

## Background of the Invention

Displays used in avionics, defence, and mission-critical applications are required to operate in both:

- Day mode, with high luminance output, and

- NVG mode, with controlled luminance and spectral compatibility

Conventional implementations typically use:

- Separate PCBs for day and NVG LEDs, or

- Dedicated backlight assemblies for direct and edge lighting schemes

Such approaches lead to:

- Increased PCB area and component count

- Complex mechanical layouts

- Challenges in maintaining luminance uniformity

- Redundant hardware for different lighting modes

- Increased cost, weight, and design complexity

There exists a need for a unified backlight scheme that supports both day and NVG modes using a single PCB and shared real estate, while being adaptable to both direct and edge lighting configurations.

## Summary of the Invention

The present invention discloses a backlight array architecture in which day LEDs and NVG LEDs are mounted on the same PCB and arranged in a defined array pattern with alternate switching capability.

The invention enables:

- Selective activation of day LEDs or NVG LEDs

- Maintenance of luminance uniformity in both modes

- Use of the same PCB layout for direct lighting and edge lighting schemes

- Preservation of identical mechanical footprint and real estate for both operating modes

The alternate switching scheme ensures that illumination characteristics remain consistent, irrespective of the lighting mode or backlight configuration.

## Detailed Description of the Invention

### LED Array Configuration

In one embodiment, the backlight comprises a single PCB populated with:

- A plurality of day LEDs, and

- A plurality of NVG-compatible LEDs

The LEDs are arranged in an interleaved or alternating pattern across the PCB, such that:

- Day LEDs and NVG LEDs are spatially distributed to ensure uniform light output

- The physical layout remains identical for both modes

### Alternate Switching Scheme

The LED array is configured for alternate electrical switching, wherein:

- In day mode, the day LEDs are activated while the NVG LEDs remain inactive

- In NVG mode, the NVG LEDs are activated while the day LEDs remain inactive

The switching scheme is designed such that:

- The illumination pattern across the display remains uniform

- No perceptible non-uniformity is introduced during mode switching

- Electrical isolation between LED groups is maintained

### Compatibility with Direct and Edge Lighting

The same PCB and LED array architecture is usable for:

- Direct lighting schemes, where LEDs illuminate the display directly from behind, and

- Edge lighting schemes, where LEDs couple light into a light guide

The LED placement, spacing, and switching logic are designed to ensure:

- Equivalent optical performance in both configurations

- No requirement for PCB redesign when changing lighting topology

- Consistent luminance distribution in both day and NVG modes

### Real Estate Optimization

A key aspect of the invention is that:

- The same PCB real estate is used for both day and NVG LEDs

- No additional PCB area is required for separate lighting modes

- Mechanical and thermal interfaces remain unchanged across modes

This allows seamless integration into compact display assemblies.

## Advantages of the Invention

- Single PCB supports both day and NVG lighting

- Reduced component count and wiring complexity

- Uniform luminance in both lighting modes

- Applicability to both direct and edge lighting schemes

- Reduced size, weight, and cost

- Simplified mechanical and optical integration